|  | Risk | Likelihood | Severity |  | Mitigation |
| --- | --- | --- | --- | --- | --- |
| A | Serious injury from cuts. Damage to hearing. Damage to eyes from sawdust etc. Burns from fuel ignition. Crush injury from timber. Injury from breaking winch rope or cable | medium | high | 1 | All operators to be trained and certificated at the appropriate level, evidence to be submitted to Town Clerk |
|  |  |  |  | 2 | Chainsaws and other specialist equipment (including saws, winches, fuel cans) brought to site by the equipment owners must be well maintained, checked to be suitable for use on site and comply with safety requirements and relevant risk assessments |
|  |  |  |  | 3 | Gloves and chainsaw approved PPE to be worn when operating saw to include safety helmet, eye protection (mesh visor or safety glasses), leg protection (chainsaw trousers), protective boots |
|  |  |  |  | 4 | Ensure saw incorporates all ten safety features and is well maintained and the chain sharpened in accordance with manufacturers guidance |
|  |  |  |  | 5 | Ensure saw is fueled away from starting and operating area |
|  |  |  |  | 6 | Inspect the work area before starting work and if necessary erect signs or undertake measures to exclude the public, if necessary use a ‘banksman’ to control public access |
|  |  |  |  | 7 | Carry the saw with the guidebar behind or scabbard fitted and wear gloves when maintaining/sharpening the saw |
|  |  |  |  | 8 | When cross-cutting, de-limbing or snedding, ensure fellow workers are a safe working distance away from you (at least 5m) |
|  |  |  |  | 9 | Make cuts in a manner that avoids chainsaw kickback and employ the appropriate cutting techniques for the timber to be cut or felled |
|  |  |  |  | 10 | Use correct manual handling techniques and handling aids eg felling lever, wedges, timber tongs, cant hook |
|  |  |  |  | 11 | Do not exceed load capacity of winch and associated equipment |
|  |  |  |  | 12 | Leave the site tidy and safe, stacks should be no more than 1m high, attach advisory signs to stacks if necessary |
| B | Serious injury from falling timber or deadwood. | medium | high | 1 | Prior to felling, check site for power lines or other overhead cables/obstructions |
|  |  |  |  | 2 | Check tree to be felled for natural lean, weight of branches dead wood in the canopy, rot or other signs of damage/decay and for nails, wire, stones where cuts are to be made |
|  |  |  |  | 3 | Check weather conditions, particularly wind, ground conditions and slope of ground |
|  |  |  |  | 4 | When felling, ensure fellow workers and obstructions are a safe working distance away from you (at least two tree lengths) |
|  |  |  |  | 5 | Deal with hung up trees immediately if possible. If hung up trees have to be left for any time, take measures to exclude public eg hazard tape |

Black type is for chainsaw operators

Blue italic is for all people involved in tree felling (including helping chainsaw operators with trees once felled)

I have read and understood this Risk Assessment and will comply with the above mitigations wherever possible in order to reduce risk to myself and others.

Name					Signature				Date

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____