|  | Risk | Likelihood | Severity |  | Mitigation |
| --- | --- | --- | --- | --- | --- |
| A | Injury to operator and to others nearby | medium | high | 1 | All operators to conduct a driving test to include: correct starting and stopping, operation of clutch leaver/deadman’s handle, use of 3 forward and reverse gears, steering/turning, use on slopes, loading and tipping procedure |
|  |  |  |  | 2 | Operator must wear safety footwear |
|  |  |  |  | 3 | Inspect the machine before and after use, check for misalignment or binding of moving parts, breakage of parts, damage to tracks and any other condition that may affect the machine’s operation. If damaged, have the machine repaired before use. |
|  |  |  |  | 4 | Thoroughly inspect the area to be worked, keep the working area clean and free of debris to prevent tripping. Plan the route in advance. Ensure there is sufficient petrol for the journey. |
|  |  |  |  | 5 | Disengage clutch lever before starting the engine |
|  |  |  |  | 6 | Start the engine carefully according to instructions and with feet well away from the moving parts |
|  |  |  |  | 7 | Always hold the unit with both hands when operating. Keep a firm grip on the handlebars. Be aware that the machine may unexpectedly bounce upward or jump forward if the machine should strike obstacles |
|  |  |  |  | 8 | Walk, never run with the machine |
|  |  |  |  | 9 | Do not overload the machine capacity. Drive at a safe speed, adjusting the speed to the slope of the land, the surface conditions of the road, and the weight of the load |
|  |  |  |  | 10 | Use extreme caution when in reverse or pulling the machine towards you |
|  |  |  |  | 11 | On soft ground or slopes, drive in first gear. Do not rapidly accelerate, turn sharply or stop. Avoid long reverses on slopes |
|  |  |  |  | 12 | When moving over a slope always make certain that the weight is evenly balanced. Always move in directions parallel with the slope (up or down). Do not change gear on slopes |
|  |  |  |  | 13 | Never operate the machine on slopes where angle is steeper than 20° |
|  |  |  |  | 14 | Pay the utmost attention when working on wet or frozen ground as the machine may skid |
|  |  |  |  | 15 | Take great care operating the machine in confined areas (eg tool store) where there may be a risk of crushing the operator between the machine and another object |
|  |  |  |  | 16 | When tipping the load from a dumper, the centre of gravity will change continuously, and the ground conditions will be essential for the stability of the machine. There are special hazards for dumpers working on soft ground and when the load is sticking to body e.g. wet clay |
|  |  |  |  | 17 | Never place any part of your body where it would be in danger if movement should occur during operation, maintenance, repairing or moving |
| B | Injury to fellow workers and general public | low | high | 1 | Keep all bystanders, children, and pets at least 20 metres away. If you are approached, stop the unit immediately, do not pass people or pets on a narrow path |
|  |  |  |  | 2 | Do not climb on dump box and never carry passengers |
|  |  |  |  | 3 | Never park the machine in a place with unstable ground which could give way, particularly when it is full |
|  |  |  |  | 4 | Never leave the operating position when the engine is running |
|  |  |  |  | 5 | Never leave the machine unattended on site except if it has been physically disabled or locked away |
|  |  |  |  | 6 | Exercise extreme caution when operating on or crossing gravel drives, paths, or roads. Stay alert for hidden hazards or traffic |
| C | Damage to machine | medium | low | 1 | If possible, avoid driving on pebbly river bed, crushed stone terrains, steel concrete, stumpy field, logs etc., since such operation causes breakage to, or shortens the life span of, tracks |
|  |  |  |  | 2 | Minimize sharp turns, avoid turns sharper than 90° wherever possible |
|  |  |  |  | 3 | Note ‘time running’ into the machine log at the end of use, this is important regarding the service interval |

I have read and understood this Risk Assessment and will comply with the above mitigations wherever possible in order to reduce risk to myself and others.

Name					Signature				Date

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____

____________________________	_______________________	____/____/_____